# Lane Edge Detection on Nexys A7 / Artix-7 FPGA Using Sobel Filter

# 1. Project Concept

Concept: This project implements a simple lane-edge detection system on an FPGA using VHDL. A road image is stored inside the FPGA as a grayscale ROM image. The FPGA reads the image pixel by pixel, processes it with a 3 x 3 Sobel filter, and displays either the original image or the Sobel edge-detected output on a VGA monitor.

Why ROM input is used: A live camera is not used in this version. Instead, a fixed road image is stored in image ROM. This keeps the project easier to implement, test, and explain.

Image resolution: The stored image is 160 x 120 pixels. The VGA output is 640 x 480 pixels, so the image is scaled by a factor of 4 to fill the display.

# 2. Design Source Files

The VHDL design source files were renamed as follows:

| VHDL file | Purpose | Beginner explanation |
| --- | --- | --- |
| Lane_Detection_using_Sobel_Filter_top.vhd | Top-level VHDL file | Connects all other modules and board I/O. |
| VGA_Display_640x480.vhd | VGA timing generator | Creates x/y position, pixel_ce, visible area, Hsync and Vsync. |
| image_rom_160x120.vhd | Image ROM | Stores the grayscale road image as pixel values. |
| Sobel_Filter_3x3.vhd | Sobel filter module | Detects image edges using a 3 x 3 pixel window. |
| Implemented_in_Artix7_100T.xdc | Constraints file | Maps VHDL ports to real FPGA board pins. |

# 3. Inputs and  Outputs

| Signal | Direction | Purpose |
| --- | --- | --- |
| CLK100MHZ | Input | 100 MHz clock from the Nexys A7 board |
| SW0 | Input | Selects original image or Sobel edge output |
| vgaRed[3:0] | Output | 4-bit VGA red signal |
| vgaGreen[3:0] | Output | 4-bit VGA green signal |
| vgaBlue[3:0] | Output | 4-bit VGA blue signal |
| Hsync | Output | VGA horizontal synchronization signal |
| Vsync | Output | VGA vertical synchronization signal |

# 4. Display Modes

| SW0 value | VGA output |
| --- | --- |
| 0 | Original road image |
| 1 | Sobel edge-detection output |

# 5. Block Diagram

# 6. Working Principle

VGA_Display_640x480 creates the VGA timing signals and gives the current x/y pixel position.

The x/y position is converted to a 160 x 120 image address and sent to image_rom_160x120.

image_rom_160x120 outputs one grayscale road-image pixel.

Sobel_Filter_3x3 builds a 3 x 3 pixel window and calculates the Sobel edge strength.

Lane_Detection_using_Sobel_Filter_top selects original image or Sobel output using SW0.

The selected grayscale pixel is sent to the VGA RGB outputs, together with Hsync and Vsync.

# 7. Limitation

This project is a ROM-based FPGA demonstration. It does not use a live camera. The purpose is to show how a stored road image can be processed with a Sobel filter in VHDL and displayed on a VGA monitor.